Right-click on Connections, in the Workspace window in DaqFactory, to create a new connection. Enter an arbitrary name, then enter the IP address from the list below; passwords are not required (the connection name can be anything that makes sense to you; the IP address fully specifies the remote computer).  Verify that “Full Stream” is NOT checked (this will reduce the amount of traffic on the network, and will only display channels deliberately broadcasted from each instrument)

ConnectionName	IP address		ChannelName			Conversion		Units		PI

Update prior to transit

NOy			10.43.16.49		NO_counts			(x-800)/5.5		ppbv		Tom

NOy_counts			(x-800)/5.5		ppbv

O3_counts			(x-200)/2.5		ppbv

CO2			10.43.16.26		CO2_mV_serial		(x/0.3)			mV		Tom/Jeff

CO			10.43.16.40		C00_pmt0						ppbv		John H.

SO2			10.43.16.42		SO2_C01_fluor_int					ppbv		John H.

NMASS		10.43.16.34		counter1			(x1013/(90*4.5) 	#/cc		Chuck

WLOPC		10.43.16.62		ncoarse						um3/cc		Adam

FltSci			10.43.16.47		AOC_GpsLat						degrees	Ken

(subset shown)	AOC_GpsLon						degrees

AOC_GpsAlt						meters

AOC_Palt						meters

AOC_Ralt						meters

AOC_AmbTemp					°C

AOC_DewPtTemp					°C

AOC_H2Omr						g/kg (parts per thousand by weight)

AOC_StaticPrs					millibar

AOC_DewPtTempTDL				°C

AOC_WindDir					degrees

AOC_WindSpd					m/s

AOC_CabinPrs					millibar

AOC_GndSpd						m/s

AOC_Heading						degrees

ConnectionName	IP address		ChannelName			Conversion		Units		PI

FltSci	(cont.)		10.43.16.47		AOC_DryExtinction					Mm-1		Dan L.

Use “ConnectionName.ChannelName” to call these remote channels in your copy of DF.  For example, to display Chuck’s small particle data, add a trace called “NMASS.counter1” to your graph.

Note that it sometimes takes several attempts to successfully connect to the remote copy of DF.  Left-click on the connection name in the Workspace window, and click on “reset connection” until the Packets Received counter begins to increment.

For N-43 during ARCPAC, the gateway is 10.43.16.1

You will also need to change the IP address of your instrument computer to the appropriate address above.