Right-click on Connections, in the Workspace window in DaqFactory, to create a new connection. Enter an arbitrary name, then enter the IP address from the list below; passwords are not required (the connection name can be anything that makes sense to you; the IP address fully specifies the remote computer).  Verify that “Full Stream” is NOT checked. (This will reduce the amount of traffic on the network, and will only display channels deliberately broadcast from each instrument).

ConnectionName	IP address		ChannelName			Conversion		Units		PI

Update prior to transit

NOyO3		10.10.16.31		NO_counts			(x-800)/5.5OLD	ppbv		Tom/Ilana

NOy_counts			(x-800)/5.5OLD	ppbv

O3_counts			(x-200)/2.5OLD	ppbv

CO2CH4		10.10.16.40		CO2_ppmv_serial		(x/0.3)OLD		ppmv		Tom/Jeff

CH4_ppmv_serial		?			ppmv 		Tom/Jeff

CO			10.10.16.45		C00_pmt0			(x-1500)/29OLD	ppbv		John H.

SO2			10.10.16.42		SO2_C01_fluor_int					ppbv		John H.

NMASS		10.10.16.43		counter1			(x1013/(90*4.5) OLD	#/cc		Chuck

FliSci			10.10.16.30		AOC_GpsLat						degrees	Ken

(subset shown)	AOC_GpsLon						degrees

AOC_GpsAlt						meters

AOC_Palt						meters

AOC_Ralt						meters

AOC_AmbTemp					°C

AOC_DewPtTemp					°C

AOC_H2Omr						g/kg (parts per thousand by weight)

AOC_StaticPrs					millibar

AOC_DewPtTempTDL				°C

AOC_WindDir					degrees

AOC_WindSpd					m/s

AOC_CabinPrs					millibar

AOC_GndSpd						m/s

AOC_Heading						degrees

ConnectionName	IP address		ChannelName			Conversion		Units		PI

FliSci	(cont.)		10.10.16.30		AOC_AMS_Org_Tot					#/mg		Ann

AOC_NO3						ppbv		Steve

AOC_N2O5						ppbv		Steve

AOC_NO2_1						ppbv		Steve

AOC_NO2_2						ppbv		Steve

AOC_NOx						ppbv		Steve

AOC_Ox						ppbv		Steve

Use “ConnectionName.ChannelName” to call these remote channels in your copy of DF.  For example, to display Chuck’s small particle data, add a trace called “NMASS.counter1” to your graph.

Note that it sometimes takes several attempts to successfully connect to the remote copy of DF.  Left-click on the connection name in the Workspace window, and click on “reset connection” until the Packets Received counter begins to increment.